Normalization in two-ray approximation model of 5D/629

Ivica Stevanovic, OFCOM, 30 October 2025

Problem statement

Using clutter losses lcld and lclg  from Rec. ITU-R P.2108-2 §3.3 directly in the two-ray sum equation from 5D/629

pr=pt∙gtcldθ,φlcldθ,φ+gtclg-θ,φlclg-θ,φ∙grloss

fails the consistency check for omnidirectional transmitters (where gtcld=  gtclg=gt ): the sum would imply

gtlcld =gt 1lcld+1lclg

which contradicts the interpretation of lcld as the (multi-ray) clutter loss.

Normalization approach

Let lcld and lclg be the clutter losses taken from P.2108 (as originally computed). Introduce a scalar normalization factor α so that the inverse clutter loss contributions are scaled and the omnidirectional limit is recovered:

gtlcld=gtα1lcld+1lclg .

Solving for α gives

α(θ,φ)=1 1+lcld(θ,φ)lclg(-θ,φ) .
Final modified two-ray formula

Using the normalization α(θ,φ), the two-ray approximation becomes

pr=pt∙α(θ,φ)gtcldθ,φlcldθ,φ+gtclg-θ,φlclg-θ,φ
with

α(θ,φ)=1 1+lcld(θ,φ)lclg(-θ,φ) 
Comments

- This choice of α guarantees that for omnidirectional transmitters the bracketed factor reduces to 1/lcld, i.e. the original P.2108 clutter loss is recovered.

- α depends on the two P.2108 losses and thus adapts the relative weight of the direct and reflected clutter contributions smoothly as the antenna pattern varies (directional-to- omnidirectional).

- This represents a simplified linear normalization. While other normalization approaches (e.g., based on energy conservation) may be more accurate, the differences compared to the case without normalization are small, so the present approach can be considered sufficiently accurate.

- Formally, the same normalization needs to be applied to the LoS case. However, that does not seem to be necessary as in this case lclg≫lcld⇒ α≈1.